MINI PIANO USING ARDUINO

Course: workshop 
Submitted By: Vinod kumar
Department: computer system engineering
Cms: 133-25-0026
Submitted To: irfan younis mughal
Session: 2k25

## 2. Acknowledgment

I would like to thank my instructor for providing guidance throughout this project. I am also thankful to my classmates and friends for their support. This project helped me learn practical skills such as circuit building, Arduino programming, and teamwork.

## 3. Introduction

The Mini Piano Project is a simple electronic musical instrument built using an Arduino, 8 push buttons, and a buzzer. Each push button represents a musical note. When a button is pressed, the Arduino generates a specific frequency that produces a tone through the buzzer.
This project demonstrates basic electronic components, digital input handling, and sound generation using Arduino.

## 4. Objectives

- To learn the basics of Arduino and its digital input pins.

- To design a simple electronic musical instrument.

- To understand how sound frequencies create musical notes.

- To improve practical skills in circuit assembly and coding.

- To develop a fun and interactive hardware project.

## 5. Table of components

| Sr | Component Name | Quantity | Description |
| --- | --- | --- | --- |
| 1 | Arduino Uno | 1 | Microcontroller board |
| 2 | Push Buttons | 8 | Used to trigger musical notes |
| 3 | Buzzer / Speaker | 1 | Produces sound output |
| 4 | Resistors (10kΩ) | 8 | Pull-down resistors |
| 5 | Jumper Wires | — | For connections |
| 6 | Breadboard | 1 | Circuit assembly |
| 7 | USB Cable | 1 | For uploading code |

## 6. Circuit Diagram (Description)

- Each of the 8 push buttons is connected to digital pins 2–9 on the Arduino.

- One side of each button goes to 5V.

- The other side goes to the Arduino pin and also to a 10kΩ resistor → GND (pull-down setup).

- The buzzer is connected to pin 10 and GND.

## 7. Working Principle

- When a button is pressed, it sends a HIGH signal to the Arduino pin.

- Arduino reads which button is pressed using digitalRead().

- The program plays a specific tone frequency for each button.

- The buzzer produces sound based on the tone() function.

- Releasing the button stops the sound.

## 8. ardiuno code

## 12. Conclusion

This project successfully demonstrates how an Arduino can be used to create a mini piano using push buttons and a buzzer. Through this project, I learned circuit designing, coding basics, and how musical notes work. The project is simple but highly educational for beginners.